6. Настройка туннеля

int tunnel.1

ip addr 10.10.10.1/30

ip tunnel 172.16.1.2 172.16.2.2 mode gre

7. Настройка OSPF

router ospf 1

passive-interface default

no passive-interface tunnel.1

network 10.10.10.0/30 area 0

network 192.168.100.0/27 area 0

network 192.168.200.0/24 area 0

network 192.168.99.0/29 area 0

(network 10.10.10.0/30 area 0)

(network 192.168.0.0/28 area 0)

int tunnel.1

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 P@ssw0rd

8. Настройка NAT

| int isp | int 100 | int 200 | int 999 |
| --- | --- | --- | --- |
| ip nat outside | ip nat inside | ip nat inside | ip nat inside |

ip nat pool VLAN100 192.168.100.1-192.168.100.30

ip nat pool VLAN200 192.168.200.1-192.168.200.254

ip nat pool VLAN999 192.168.99.1-192.168.99.6

ip nat source dynamic inside-to-outside pool VLAN100 overload interface isp

ip nat source dynamic inside-to-outside pool VLAN200 overload interface isp

ip nat source dynamic inside-to-outside pool VLAN999 overload interface isp

9. Настройка DHCP

ip pool VLAN200 192.168.200.2-192.168.200.254

dhcp-server 1

pool VLAN200 1

mask 24

gateway 192.168.200.1

dns 192.168.100.2

domain-name au-team.irpo

int 200

dhcp-server 1

10. Настройка DNS

systemctl enable --now dnsmasq

vim /etc/dnsmasq.conf

domain=au-team.irpo
expand-hosts
listen-address=192.168.100.2,127.0.0.1
server=77.88.8.7

address=/hq-rtr.au-team.irpo/172.16.1.2
address=/br-rtr.au-team.irpo/172.16.2.2
address=/hq-srv.au-team.irpo/192.168.100.2
address=/hq-cli.au-team.irpo/192.168.200.2
address=/br-srv.au-team.irpo/192.168.0.2
address=/docker.au-team.irpo/172.16.1.1
address=/web.au-team.irpo/172.16.2.1

systemctl restart dnsmasq

vim /etc/resolv.conf

nameserver 192.168.100.2

search au-team.irpo

ip domain-name au-team.irpo
ip name-server 192.168.100.2